ACL

The 'ip access-list' is a global configuration mode command. To create a standard access list, it uses the following syntax.

Router(config)# ip access-list standard ACL_#

In the above syntax, the ACL_# is the name or number of the standard ACL. When you hit the enter key after entering this command, the command prompt changes and you enter standard ACL configuration mode.

Router(config-std-acl)#

In standard ACL configuration mode, you can use the following syntax to create statements.

Router(config)# ip access-list standard ACL_name

Router(config-std-acl)# permit|deny source_IP_address [wildcard_mask]

An ACL does nothing until it is applied to an interface. To apply a standard ACL to an interface, enter the interface configuration mode of the interface and use the following command.

Creating a standard ACL

Access the command prompt of Router0 and run the following commands.

Router(config)# interface type [slot_#]port_#

Router(config-if)# ip access-group ACL_# in|out

Router>

Router>enable

Router#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Router(config)#ip access-list standard BlockStudents

Router(config-std-nacl)#deny 10.0.0.0 0.255.255.255

Router(config-std-nacl)#permit any

Router(config-std-nacl)#exit

Router(config)#interface gigabitethernet 0/0

Router(config-if)#ip access-group BlockStudents out

Router(config-if)#exit

Router(config)#exit

Router#

Deleting a standard ACL

To delete a standard ACL, use the following command in global configuration mode.

Router(config)no ip access-list standard ACL_#